This report reflects a BROKEN state of the hardware at the time of writing. The drivetrain was subsequently found FULLY FUNCTIONAL: a dedicated motor-health test showed both motors spin, regulate speed within ~2%, and hold deterministic, opposite CW/CCW direction across repeated runs, with encoder counts matching theory (1336 counts/rev). The true root cause of the earlier erratic behaviour was MARGINAL ENCODER FEEDBACK (wiring/connectors), NOT a driver or motor defect, and NOT a reason to buy new controllers. The RMCS-2303 drivers and the motors are retained and working. The analysis below is kept only as a record of the investigation.

*** SUPERSEDED - DO NOT ACT ON THE CONCLUSIONS BELOW ***

Rover Drivetrain Fault Analysis Report

Hardware Fault Isolation and Replacement Justification

Differential-Drive Rover — Motor & Motor-Driver Subsystem

Prepared by: Akhilesh Jaikumar, RP Tech India    |    Date: 13 July 2026

# 1. Executive Summary

The rover’s drivetrain (two DC servo motors driven by Rhino RMCS-2303 Modbus servo drives) fails to execute motion commands reliably. A systematic, layer-by-layer fault isolation campaign was carried out across the complete control chain — ROS 2 teleoperation on the Jetson AGX Orin, micro-ROS firmware on the ESP32, the Modbus ASCII link, the RMCS-2303 drives, and the motors themselves.

The software stack was instrumented, corrected where defects were found, and then validated end-to-end with measured evidence at every layer. The remaining failures were subsequently reproduced using a minimal standalone test firmware that bypasses the entire software stack (no ROS 2, no micro-ROS) — driving the RMCS-2303 units directly over Modbus with checksum-validated, acknowledged frames. The faults persisted unchanged.

This isolates the remaining failures to the motor / motor-driver hardware. Four distinct hardware faults were documented:

Non-deterministic direction execution: the right RMCS-2303 executes the identical, acknowledged direction command in different physical directions on different runs. Reproduced across dozens of controlled trials, with two different motors attached.

Complete drive failure (right motor, original): the drive reported the enabled/running state while the motor produced zero motion — traced to a degraded motor power connection.

Encoder failure (left motor, original): position feedback returned physically impossible values (deltas in exact multiples of 65,536, counting in one direction regardless of rotation) — the signature of a dead quadrature channel. The motor has since been replaced.

Mismatched replacement pair: the replacement left motor’s encoder produces approximately 230 times fewer counts than the right motor at the same commanded speed, making matched differential drive impossible with the current mixed pair.

Recommendation: procure a matched pair of identical servo motors (same model, gear ratio, and encoder specification) and replace the RMCS-2303 drives — at minimum the right-hand unit — with drives that provide deterministic, absolute direction control. No further software work can restore reliable operation on the current hardware.

# 2. System Under Test

Control chain: ROS 2 Humble teleoperation node (Jetson AGX Orin) → /cmd_vel → micro-ROS agent (serial, 115200 baud) → ESP32 firmware → Modbus ASCII at 9600 baud on two independent UARTs → RMCS-2303 DC servo drives (left: slave ID 7, right: slave ID 2) → DC servo motors with quadrature encoders.

# 3. Software Stack: Verified Correct

During the investigation, defects were found in the firmware and teleoperation code. Each was fixed and its fix verified by measurement. This due diligence matters for the conclusion: after these corrections, every software layer was independently validated, and the faults that remain are demonstrably outside software control.

## 3.1 Defects found and fixed (with verification)

| Defect | Fix | Verification |
| --- | --- | --- |
| Stop command used 0x0100, which per the RMCS-2303 manual only selects Mode 1 and does not stop a running motor | Use 0x0000 (true stop) preceded by a zero-speed write | Motors verifiably stop on command; confirmed via encoder feedback |
| Firmware re-sent identical Modbus commands every 200 ms, blocking the micro-ROS executor ~180 ms each cycle | Skip transmission when the command is unchanged | Command latency reduced to one 200 ms cycle; session stable |
| Agent liveness ping transmitted on every loop iteration, competing with incoming commands on the same UART | Ping throttled to once per 2 s | No further session drops |
| 2-second blocking delay on direction reversal starved the micro-ROS session | Non-blocking stop-and-return; next periodic command restarts cleanly | Reversals execute reliably at 5 Hz teleop rate |

## 3.2 End-to-end validation of the command path

With fixes applied, the full ROS-to-firmware path was validated by commanding all four motion primitives and reading back the firmware’s applied wheel commands. The differential kinematics are exact and symmetrical:

| Command | Left wheel (ticks) | Right wheel (ticks) | Assessment |
| --- | --- | --- | --- |
| Forward | +6,062 | +6,062 | Correct — equal forward |
| Backward | −1,436 | −1,436 | Correct — equal reverse |
| Turn left | −2,561 | +2,561 | Correct — symmetric opposite |
| Turn right | +2,143 | −2,143 | Correct — symmetric opposite |

In addition, real encoder telemetry was implemented (Modbus function 03 reads of the drives’ position registers, checksum-validated) and published at 2 Hz. This provides ground-truth measurement of actual motor motion, independent of what was commanded — the instrument used for all hardware evidence below.

# 4. Hardware Fault Evidence

## 4.1 Fault 1 — Right drive: commands accepted, zero motion

With the software stack removed and a standalone diagnostic firmware driving the right RMCS-2303 directly: every Modbus frame was acknowledged (17-byte ACK), the drive’s mode register confirmed the enabled state (0x1601), the speed target register read back the commanded 50 RPM — and the motor did not move. Actual-speed register: 0. Position register: frozen for the full 4-second drive window.

| Time | ACT_SPD (reg 24) | Position (reg 20) | Mode state (reg 3) |
| --- | --- | --- | --- |
| 0 ms | 0 | 1 (frozen) | 0x1601 (enabled) |
| 1200 ms | 0 | 1 (frozen) | 0x1601 (enabled) |
| 2400 ms | 0 | 1 (frozen) | 0x1601 (enabled) |
| 3600 ms | 0 | 1 (frozen) | 0x1601 (enabled) |

The fault cleared after the motor’s power wiring was re-made — consistent with a degraded high-current connection. A drivetrain whose power path fails intermittently with handling is not field-reliable.

## 4.2 Fault 2 — Left motor (original): encoder returning impossible data

The original left motor’s position feedback exhibited deltas that are exact multiples of 65,536 (e.g. 7,143,424 = 109×65,536; 1,703,936 = 26×65,536), meaning the low 16 bits of the position never advanced while the high word churned — physically impossible for quadrature counting. The count direction was also positive regardless of commanded rotation, and magnitudes varied randomly between 0.5M and 7M counts for identical 2-second runs. This is the classic signature of a dead quadrature channel. A servo drive cannot regulate speed or direction with this feedback. The motor was replaced.

## 4.3 Fault 3 — Non-deterministic direction execution (right drive)

This is the decisive defect. A standalone probe firmware (no ROS, no micro-ROS) executed a fixed sequence of direction commands — 0x0101 and 0x0109, the two documented run commands — with a full stop between every step. Every frame was LRC-checksummed and acknowledged by the drive. The physical rotation direction was measured from the encoder for each step. The identical command sequence produced different direction patterns on different passes:

| Pass | Command sequence | Measured direction signs | Consistent? |
| --- | --- | --- | --- |
| Right, pass 1 | 01, 01, 09, 09, 01, 09, 01 | − − + − + − + | No |
| Right, pass 2 | 01, 01, 09, 09, 01, 09, 01 | + + + + + − − | No |
| Right, pass 3 (new motor) | 01, 01, 09, 09, 01, 09, 01 | − − − + − − + | No |
| Right, pass 4 (new motor) | 01, 01, 09, 09, 01, 09, 01 | + − − − − − + | No |
| Left (replacement motor) | 01, 01, 09, 09, 01, 09, 01 | + + − − + − + | Yes — textbook |

Key observations:

The same acknowledged command word yields opposite rotation directions across runs on the right drive. No open-loop control scheme can function on such a device.

The behaviour persisted after the right motor was replaced — eliminating the motor and implicating the drive’s direction-latching logic.

The replacement left motor on the left drive produced the exact textbook pattern (13 consecutive consistent steps), demonstrating that the test method itself is sound.

Speed regulation on the right drive is meanwhile excellent (run magnitudes repeatable within ~2%: ≈770k counts one way, ≈810k the other) — only direction execution is defective, further pointing to a drive logic fault rather than wiring.

## 4.4 Fault 4 — Mismatched motor pair after replacement

The replacement left motor is not the same model as the right motor. At the identical commanded speed and run time, the left encoder registers ≈3,500 counts while the right registers ≈800,000 counts — a factor of ≈230. Both drives are configured for 334 encoder lines per rotation; at most one motor can actually match that. Consequently the two drives’ closed-loop “RPM” units refer to different physical wheel speeds, and the rover cannot drive straight. Differential drive fundamentally requires a matched pair of motors with identical gearing and encoder specification.

# 5. Why Software Cannot Compensate

Direction randomness (Fault 3) originates inside the drive after a command is verifiably received and acknowledged. Software has no register through which to command direction deterministically; an encoder-feedback correct-and-retry loop was prototyped, but it is a workaround that adds 200–600 ms of correction latency per manoeuvre, produces visible wrong-direction jerks, and depends on the same defective latch it is trying to fix.

The encoder mismatch (Fault 4) cannot be calibrated away: even with per-side scaling, the two motors differ in gearing, dynamics, and resolution, so speed tracking, acceleration, and odometric accuracy will never match between the two wheels.

The intermittent power-path failure (Fault 1) and the failed encoder (Fault 2) are plain hardware wear-out/defect items.

# 6. Recommendation

Procure one matched pair of identical DC servo gear-motors (same model, same gear ratio, same encoder lines-per-rotation), specified to the rover’s load. This resolves Faults 2 and 4 and retires the aged, intermittent units.

Replace the right-hand RMCS-2303; given the documented direction-latch behaviour and to keep the pair identical, replacing both drives is advised. Select drives with an absolute, documented direction command and readable fault status.

The complete software stack (teleoperation, micro-ROS firmware, Modbus layer, odometry, SLAM launch) is validated, instrumented with real encoder telemetry, and committed to version control. It will operate the new hardware as-is after a one-time direction/scale calibration, for which tooling is already in place.

Every additional day of accommodating the current hardware is engineering time spent working around components that have been demonstrated — with checksummed, acknowledged, software-independent measurements — to be defective or mismatched. Replacement is the economical path.

# Appendix A — Drive configuration parity

Both drives hold identical, healthy configuration (read back over Modbus), ruling out configuration divergence as a cause:

| Register | Left drive (ID 7) | Right drive (ID 2) |
| --- | --- | --- |
| P gain (reg 4) | 22 | 22 |
| I gain (reg 6) | 16 | 16 |
| FF gain (reg 8) | 32 | 32 |
| Encoder lines (reg 10) | 334 | 334 |
| Acceleration (reg 12) | 20000 | 20000 |

Test artefacts: standalone diagnostic and direction-probe firmware (motor_test environments diag / dirprobe), encoder telemetry firmware (/encoder_ticks), and raw logs are preserved in the project repository (github.com/NightFury09/ExpoAssistant).